Operation and installation manual

Bulk Bag Unloading system

MODÈLE/MODEL BBUFL

| ITEM | DESCRIPTION | DÉSIGNATION |
| --- | --- | --- |
| 1 | LOWER STRUCTURE | STRUCTURE INFÉRIEURE |
| 2 | UPPER STRUCTURE | STRUCTURE SUPÉRIEURE |
| 3 | SLIDING LEGS | PIED COULISSANT |
| 4 | GASKET | JOINT D’ÉTANCHÉITÉ |
| 5 | GASKET FLANGE (LONG) | BRIDE DU JOINT D’ÉTANCHÉITÉ(LONGUE) |
| 6 | GASKET FLANGE (SHORT) | BRIDE DU JOINT D’ÉTANCHÉITÉ(COURTE) |
| 7 | GASKET FLANGE (ROND) | BRIDE DU JOINT D’ÉTANCHÉITÉ(RONDE) |
| 8 | CYLINDER BRACKET | SUPPORT CYLINDRE MASSEUR |
| 9 | HOPPER WITH ACCESS DOOR | TRÉMIE AVEC PORTE D’ACCÈS |
| 10 | BULK BAGS STRUCTURE | STRUCTURE SUPPORT SUPER SAC |
| 11 | GUIDE | GUIDE POUR STRUCTURE SUPPORT SUPER SAC |
| 12 | OLI VIBRATOR | VIBRATEUR OLI |
| 13 | HITCH PIN | TIGE CYLINDRIQUE |

MODÈLE / MODEL BBUH

| ITEM | DESCRIPTION | DÉSIGNATION |
| --- | --- | --- |
| 1 | LOWER STRUCTURE | STRUCTURE INFÉRIEURE |
| 2 | UPPER STRUCTURE | STRUCTURE SUPÉRIEURE |
| 3 | SLIDING LEGS | PIED COULISSANT |
| 4 | GASKET | JOINT D’ÉTANCHÉITÉ |
| 5 | GASKET FLANGE (LONG) | BRIDE DU JOINT D’ÉTANCHÉITÉ(LONGUE) |
| 6 | GASKET FLANGE (SHORT) | BRIDE DU JOINT D’ÉTANCHÉITÉ(COURTE) |
| 7 | GASKET FLANGE (ROND) | BRIDE DU JOINT D’ÉTANCHÉITÉ(RONDE) |
| 8 | CYLINDER BRACKET | SUPPORT CYLINDRE MASSEUR |
| 9 | HOPPER WITH ACCESS DOOR | TRÉMIE AVEC PORTE D’ACCÈS |
| 10 | RAIL STRUCTURE | STRUCTURE POUR MONORAIL |
| 11 | UPPER BEAM | POUTRE SUPÉRIEURE MONORAIL |
| 12 | LOWER BEAM | POUTRE INFÉRIEURE MONORAIL |
| 13 | SAFETY STOPPER | ARRÊT DE SÉCURITÉ |
| 14 | LIFTING CROSS | CROIX DE LEVAGE |
| 15 | OLI VIBRATOR | VIBRATEUR OLI |
| 16 | HITCH PIN | TIGE CYLINDRIQUE |

TECHNICAL DATA

BAGS size :

Maximal vertical dimension : 72"

Maximal width : 46,5"

Discharge spout diametre : 18"

Discharge spout lenght untill 24"

Loading capacity of 2 metric ton;

- Heavy duty seal supporting bag for a perfect sealing

- Access door with quick clips. (No tools required)

- Flexible conduct connects to universal adapter. This can then be connected to a rotary valve, a screw feeder or any other equipment required by the process.

- Lifting cross for easy grab with a fork lift.

- Built of stainless steel 304 or carbon steel.

- Easy assembly requires only bolting.

- Adjustable legs for the lower structure and upper structure, 7 positions distanced by 4 inches each for a total of 56 inches of adjustment. Holding through cylindrical rods.

- Adjustment to allow the emptying of bags of different sizes

OPTIONS :

- Diaphragm valve (IRIS Valve) allows the opening of the bag while having the access door closed

- Motorized hoist and trolley for operation without forklift;

- Dust collecteur connection;

- Level detector plug;

- installation on load cells.

Advantages :

- Fast assembly and disassembly of parts and accessories

- 2 different models, used upon available resources. Fork lifts access model or hoist and trolley loading.

- Operators are protected against bags falls with rigid chassis support.

- Door provides easy access to the interior of the hopper.

- Massing cylinders improve the emptying of bulk bags.

Contraindications :

- Bulk bag unloading system is not designed to be in contact with hazardous materials. Where appropriate, it is mandatory to inform the manufacturer.

Explosive, flammable, toxic and / or harmful materials are considered dangerous materials

- Working temperature : -20°C / 55°C

Accessories :

- The pneumatic actuators provide a massage to the lower part of the bulk bag in situations where slightly fluid materials do not flow easily. The progressive movement of the massing cylinders make a "V" at the bottom of the bag, allowing the removal of dead points and thus a more efficient flow of material.

Construction :

- Carbon steel or stainless steel.

Foundations

TOTAL DYNAMIC VERTICAL LOAD:          4100 daN

TOTAL HORIZONTAL LOAD:                      Mf max 500 daN * m

Anchorage system not provided. Use anchor bolts adapted Ø ½-13UNC. We recommend using grade 8 bolts, since they bear the greatest loads of the system.