# SPECIFICATIONS CONVEYING EDUCTOR

MODEL L.E.1

- Conveying eductor machined from bar rated for a capacity of ______#/hour of ________(name material to be conveyed) over an horizontal conveying distance of _____ft and a vertical conveying distance of _____feet with ____ 90 degrees long radius elbows.

- The material inlet must be in Stainless Steel;

- The nozzle must be carbon steel painted with threads to match material inlet TEE.

- Nozzle must be machined to operate at a pressure of _____PSI at a flow rate of ______SCFM.  Mach number across the nozzle should be between 0,27 and 0,40.

- The eductor must be designed to operate at an altitude of _____Feet above sea level;

- The venturi part must be made of painted carbon steel.

- It shall be threaded and machined to fit a conveying line of _____inches inside diameter and _____inches outside diameter.

- The venturi section must be replaceable without changing the material inlet TEE.

- Paint Orange LEAP industrial enamel.

# Application data sheet – Pneumatic conveying eductor

Please fill this form with all necessary information for our engineers to set a price for the realization of your project.

PLEASE FAX TO 450-462-0756 THANK YOU

| Client name: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Contact agent : |  |  |  |  |  |  |
| Phone number: |  |  |  |  |  |  |
| E-Mail: |  |  |  |  |  |  |

| Material(s) |  |  |  | Bulk density |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name: |  |  |  | Moisture content (%) |  |  |
|  |  |  |  | Temperature |  |  |
|  |  |  |  | Hygroscopic |  |  |
|  |  |  |  | Abrasive |  |  |
|  |  |  |  | Corrosive |  |  |
|  |  |  |  | Toxic |  |  |
|  |  |  |  | Explosive |  |  |
|  |  |  |  | Free flowing |  |  |
|  |  |  |  | Aeration |  |  |
|  |  |  |  | Dimension (µm) |  |  |

| Conveying data |  |  |  | Conveying rate (lbs/hr.) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal distance (ft.) |  |  |
|  |  |  |  | Vertical distance (ft.) |  |  |
|  |  |  |  | Number of long radius elbow(s) |  |  |
|  |  |  |  | Altitude above sea level |  |  |

| Fabrication material |  |  |  | Carbon steel |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Stainless Steel 304 |  |  |
|  |  |  |  | Stainless Steel 316 |  |  |
|  |  |  |  | Carbon steel ceramic lining |  |  |

| Electrical power source |  |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cycle |  |  |
|  |  |  |  | Volts |  |  |

| Miscellaneous |  |  |  | Available air pressure |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Available line (insideØ) |  |  |
|  |  |  |  | Paint finish specifications |  |  |
|  |  |  |  | Weld specifications |  |  |
|  |  |  |  | Motive fluid type |  |  |
|  |  |  |  | Air |  |  |
|  |  |  |  | N2 |  |  |
|  |  |  |  | Others |  |  |